Title

Authors and Affiliations

Abstract

- Specifications Table

- Background & Summary

- Value of the Data

- Methods

  - Data sources

The dataset was assembled from multiple crystallographic and powder X-ray diffraction data sources, including the RRUFF Project, OPXRD, and the Crystallography Open Database (COD). Source-specific identifiers were retained during data ingestion to preserve data provenance and enable records in the integrated dataset to be traced back to their original sources.

RRUFF data were obtained from the RRUFF Project，Five RRUFF powder-diffraction source archives were used: DIF, Refinement Data, Refinement Output Data, XY Processed, and XY RAW. The downloaded source files contained mineral-associated diffraction information, including X-ray diffraction profiles, DIF records, and refinement-related data. The RRUFF records included both experimentally measured powder diffraction patterns and calculated diffraction profiles associated with mineral samples. Original RRUFF sample identifiers were retained throughout subsequent processing.

OPXRD data were obtained from [exact dataset publication/repository and version to be added]. Powder diffraction records and their associated metadata were extracted from the source data while preserving the original OPXRD identifiers for provenance tracking.

Crystal structure information was additionally obtained from the Crystallography Open Database (COD) [version or access date to be added]. Original COD identifiers were retained and linked to the corresponding normalized structure records in the integrated dataset.

Detailed procedures for parsing, standardizing, linking, and storing these source records are described in the following sections.

  - Data standardization

  - Crystal structure representation

  - Powder diffraction representation

  - Entity linking and identifiers

  - Database implementation

- Data Record

- Technical Validation

- Usage Notes

- Data Availability

- Code Availability

- References

- Author Contributions

- Competing Interests

- Acknowledgements

- Funding

- Disclosure of AI use